RNA extraction: Grape protocol (Hemanth et al., 2008. EJB. 11(3):1-5.

- Grind 1.5 gr of caladium tissue on mortar and pestle.

- Add 0.03 gr of polyvinylpyrrolidone (PPPV) to a 20 ml tube, frozen the tube in N2liq and add the tissue.

- Pre-warmed extraction buffer (4 M guanidine thiocyanate, 100 mM Tris-HCl, pH 8.0, 25 mM sodium citrate pH 8.0, 0.5% N- Lauryl sarcosine) add 2 mL of 5M NaCl and 200 L B-Mercaptoethanol.

- Add 20 ml of extraction buffer to the tissue.

- Vortex 1 min and incubate on ice 5 min. Add 1 vol of Chloroform: Isoamyl Alcohol 24:1 and hand shake vigorously 5 to 10 min.

- Centrifuge at 14,000 xg for 15 min at 4°C.

- Repeat step 3 and 4.

- Transfer the supernatant to a fresh tube and add 0.1 vol of 5M NaCl and 1 vol. of Isopropanol. Incubate at -20°C over night.

- Pellet at 14,000 xg for 20 min at 4°C. Wash pellet with an equal volume of 75% EtOH and centrifuge at 8,000 xg for 10 min. Discard the aqueous phase and dry.

- Re-suspend in 2 mL of H2O DECP treated and incubate for 5 to 10 min at 65°C.